Integration Proposal

CANopenNode for UC2-ESP32

Replacing Custom ISO-TP with a Standardized CANopen Stack

Prepared for: openUC2 / youseetoo

March 2026

Approach: Incremental Integration — No Rewrite

# 1. Executive Summary

This document proposes incrementally integrating the CANopenNode stack into the existing UC2-ESP32 firmware to replace the current custom ISO-TP CAN bus implementation. The goal is not a rewrite — it is a targeted swap of the CAN transport layer behind your existing Module interface, gaining standardized device management, a proper Object Dictionary (OD), built-in NMT (Network Management) lifecycle, and — critically — a standards-based path to CAN-bus firmware updates (SDO block transfer).

The existing architecture (ModuleController, JSON REST API over Serial/WiFi, PinConfig system) remains untouched. The CAN layer changes from custom ISO-TP framing to CANopen PDOs/SDOs, routed through a thin adapter that maps between your JSON commands and the OD.

# 2. Current CAN Bus Architecture — What Exists Today

## 2.1 Network Topology

Your current system follows a star-over-bus topology: A Raspberry Pi (or Jetson Orin) connects via USB Serial to a CAN HAT Master ESP32S3. This master bridges serial JSON commands onto the CAN bus, where satellite ESP32S3 Xiao boards (motor controllers, LED drivers, laser modules) listen and respond. Every satellite runs the same firmware image differentiated only by a CAN node ID stored in NVS.

## 2.2 CAN Transport: Custom ISO-TP over TWAI

The CAN communication is handled by your forked ESP-CAN-ISO-TP-TWAI library (itself forked from MLeganes/arduino-CAN-ISO-TP). This implements ISO 15765-2 transport protocol (single frames, first frames, consecutive frames, flow control) on top of the ESP32’s built-in TWAI peripheral. It works, but it has several limitations:

- Custom framing: The protocol is home-grown on top of ISO-TP. There is no standard service layer — no heartbeat, no node lifecycle (NMT), no emergency objects. If a satellite crashes, the master has no built-in way to know.

- No Object Dictionary: Configuration and state are passed as raw byte blobs or JSON fragments tunneled through CAN. There is no standardized way for a tool to introspect what a node can do.

- Firmware updates over CAN are non-trivial: Your chat_can_ota.md document already recognizes this gap. ISO-TP can carry OTA payloads, but you need a custom bootloader protocol on top.

- No standard tooling: You cannot use off-the-shelf CANopen analyzers, EDS files, or Linux canopen utilities to debug the bus.

## 2.3 What Works Well (Keep It)

| Component | Assessment |
| --- | --- |

| ModuleController | Excellent modular design — keep as-is. The map<AvailableModules, Module*> pattern is clean. |
| --- | --- |
| JSON REST API | Well-understood by ImSwitch and existing Python tooling. Keep for Serial/WiFi. CAN messages should translate to/from this internally. |
| PinConfig system | Build-time board selection works well. CAN_RX/CAN_TX pins already defined per board variant. |
| TWAI driver usage | The ESP-IDF TWAI driver is the correct low-level driver. CANopenNode_ESP32 also uses it. |
| Same firmware for all satellites | Brilliant — CANopen’s node ID assignment makes this even cleaner. |
| NVS for CAN ID storage | Keep. CANopen typically stores node-id in non-volatile memory too. |

# 3. Why CANopenNode (and Not Something Else)

## 3.1 CANopenNode Overview

CANopenNode is a free, open-source (Apache 2.0) CANopen protocol stack written in ANSI C. It is the most widely ported CANopen implementation for embedded systems, with official ports to STM32, PIC, Zephyr RTOS, Linux socketCAN, and — crucially — ESP32 via the sicrisembay/CANopenNode_ESP32 component (ESP-IDF v5.2, uses TWAI driver natively).

It implements CiA 301 (the core CANopen application layer) including NMT lifecycle, SDO client/server, PDO mapping, Heartbeat producer/consumer, and Emergency objects. The code is non-blocking and designed to run in RTOS tasks — a perfect fit for the FreeRTOS environment on ESP32.

## 3.2 Comparison: Current Approach vs. CANopenNode

| Capability | Current (ISO-TP custom) | With CANopenNode |
| --- | --- | --- |
| Node lifecycle | None — manual ping/response | NMT: Boot → Pre-op → Operational → Stopped |
| Node health | No heartbeat | Heartbeat producer/consumer, configurable interval |
| Config access | Custom byte protocol | SDO read/write to OD entries — standard tooling works |
| Real-time data | Polled JSON over CAN | PDOs: event-driven, mapped, up to 8 bytes per PDO |
| Firmware update | Implemented => Streaming from master to slave, but relatively slow | SDO block transfer (CiA 301 §7.2.4.3) or LSS + custom bootloader |
| Bus debugging | Custom log parsing | Any CANopen analyzer, candump + EDS file |
| Interoperability | Only UC2 devices | Any CiA 301-compliant device |

## 3.3 Why the skpang Trainer Is a Good Starting Point

The skpang/ESP32S3_CANOpenNode_trainer project demonstrates CANopenNode running on an ESP32-S3 with the Arduino framework under PlatformIO — exactly your build environment. It provides a working CO_driver_target.h/.c for the ESP32-S3 TWAI peripheral, a minimal Object Dictionary, and a main loop showing the CANopenNode initialization and processing pattern. You can lift the driver files and OD template directly.

However, for production use, the sicrisembay/CANopenNode_ESP32 component (pure ESP-IDF) is more actively maintained and uses the official CANopenNode v4 API. Since your PlatformIO projects already use the espidf+arduino dual framework, you can include it as an ESP-IDF component alongside Arduino code.

# 4. Integration Strategy: Layer Cake, Not Rewrite

The key insight is that your existing Module system already cleanly separates hardware control from communication. We insert CANopenNode as a new communication transport alongside Serial and WiFi, not replacing them.

## 4.1 Architectural Layers (Before and After)

| Before (Current) | After (Proposed) |
| --- | --- |
| RaspberryPi ↓ USB Serial (JSON) CAN HAT Master ESP32 ↓ ISO-TP over TWAI (custom bytes) Satellite ESP32S3 nodes ↓ Module.act() / Module.get() Hardware (motors, LEDs, lasers) | RaspberryPi ↓ USB Serial (JSON, unchanged) CAN HAT Master ESP32 ↓ CANopen (NMT + SDO + PDO) Satellite ESP32S3 nodes ↓ OD ↔ Module.act() / Module.get() Hardware (motors, LEDs, lasers) |

## 4.2 The CANopen Module: A New Module Class

Create a new CANopenModule class (replacing the current CanController logic) that implements your Module interface. On the master, it acts as a CANopen NMT master and SDO client. On satellites, it acts as a CANopen NMT slave with an OD server. The module’s setup() initializes CANopenNode (CO_init, CO_CANinit, CO_NMT_init, etc.), its loop() calls CO_process() at 1ms intervals, and act()/get() translate between incoming JSON commands and OD writes/reads.

## 4.3 Object Dictionary Design for UC2

This is the core design decision. The OD maps your UC2 device capabilities to standardized CANopen indices. Here is a proposed OD layout:

| Index | Sub | Name | Type | Usage |
| --- | --- | --- | --- | --- |
| 0x1000 | 0 | Device type | U32 | CiA standard |
| 0x1017 | 0 | Heartbeat producer | U16 | ms interval |
| 0x1018 | 1–4 | Identity | U32 | Vendor/product/revision/serial |
| 0x2000 | 1–4 | Motor target pos | I32 | Steps, per axis (RPDO mapped) |
| 0x2001 | 1–4 | Motor actual pos | I32 | Steps, per axis (TPDO mapped) |
| 0x2002 | 0 | Motor command | U8 | 0=stop, 1=move, 2=home |
| 0x2010 | 1–3 | Laser PWM | U16 | Per laser channel |
| 0x2020 | 0 | LED array data | DOMAIN | SDO block for LED patterns |
| 0x2030 | 0–4 | Digital inputs | U8 | Endstop/sensor states |
| 0x2100 | 0 | FW version | STR | Read-only |
| 0x2F00 | 0–2 | OTA control | DOMAIN | Firmware update via SDO block |

Indices 0x1000–0x1FFF follow the CiA 301 standard. Manufacturer-specific entries start at 0x2000. The OD is generated from an EDS (Electronic Data Sheet) file using the CANopenEditor tool, which outputs C source code (OD.h / OD.c) that compiles directly into your firmware.

## 4.4 PDO Mapping: Lean Real-Time Communication

You mentioned wanting minimal distributed state. CANopen PDOs are perfect for this: each PDO carries up to 8 bytes of mapped OD data, sent either on change or at a fixed interval. Here is a minimal PDO mapping that covers your primary use cases:

| PDO | Direction | Mapped OD Entries | Trigger |
| --- | --- | --- | --- |
| RPDO1 | Master→Slave | 0x2000:1–2 (target X,Y pos) | On command from Pi |
| RPDO2 | Master→Slave | 0x2010:1–3 (laser channels) | On change |
| TPDO1 | Slave→Master | 0x2001:1–2 (actual X,Y pos) | Every 100ms or on change |
| TPDO2 | Slave→Master | 0x2030:0–4 (digital inputs) | On change (endstops) |

This means motor position commands are a single 8-byte CAN frame — no ISO-TP overhead, no JSON parsing on the bus. Endstop events arrive as interrupts, not polls. The master aggregates these into JSON responses for the Pi.

# 5. Firmware Updates Over CAN

This is one of your primary motivations. CANopen provides two mechanisms suitable for OTA:

## 5.1 SDO Block Transfer (Recommended)

SDO block transfer (CiA 301 §7.2.4.3) allows transferring large data blocks (your firmware binary) to a specific OD entry (e.g., 0x2F00) on a target node. The satellite receives the binary into a secondary OTA partition, verifies the CRC, and reboots. This is reliable, uses standard CANopen framing, and the master controls the flow. At 500 kbit/s CAN, a 1 MB firmware image takes roughly 25–30 seconds.

## 5.2 Proposed OTA Workflow

- Pi sends JSON command to master: {"task": "/ota_start", "node": 5, "size": 1048576, "crc32": "A1B2C3D4"}

- Master writes OTA metadata to satellite node 5 via SDO (OD 0x2F00:1 = size, 0x2F00:2 = CRC)

- Master performs SDO block transfer of firmware binary to OD 0x2F00:0 (DOMAIN type)

- Satellite writes received data to OTA partition using esp_ota_write()

- On completion, satellite verifies CRC, calls esp_ota_set_boot_partition(), and resets via NMT reset

The entire flow uses standard CANopen services. No custom bootloader protocol needed. The CANopenNode SDO server handles segmentation, flow control, and error recovery automatically.

# 6. Implementation Plan: Four Phases

## Phase 1: Proof of Concept (1–2 weeks)

- Add CANopenNode + CANopenNode_ESP32 as git submodules under lib/ or components/

- Create a new PlatformIO environment (e.g., env:xiao_canopen_test) with a minimal OD (device type, heartbeat, one custom entry)

- Implement CO_driver_target.h/.c using the TWAI driver (reference: skpang trainer or sicrisembay port)

- Verify two ESP32S3 boards can exchange heartbeats and a single SDO read/write

- Validate with Linux candump / canopen-dump on the Pi side (via the CAN HAT hardware)

## Phase 2: CANopenModule Integration (2–3 weeks)

- Create CANopenModule.h/.cpp implementing the Module interface (setup/loop/act/get)

- Design the full UC2 Object Dictionary using CANopenEditor; generate OD.h/OD.c

- Map motor target/actual positions to PDOs; map laser/LED control to SDO-writable OD entries

- On the master: implement the JSON-to-OD bridge that translates serial commands to SDO writes and PDO triggers

- Add #ifdef CAN_CONTROLLER_CANOPEN guards so the old ISO-TP path remains compilable for existing boards

## Phase 3: Firmware Update Over CAN (2 weeks)

- Implement the OTA OD entries (0x2F00 range) with a DOMAIN-type SDO callback that streams into esp_ota_write()

- Implement the master-side OTA command handler that reads a .bin file from serial or SPIFFS and performs SDO block transfer

- Test: update a satellite motor node from the Pi via JSON command through the master

## Phase 4: Migration and Cleanup (1–2 weeks)

- Update all CAN-related PlatformIO environments (can-hat-master-v2, xiao-can-slave-motor, etc.) to use the new CANopenModule

- Deprecate (but don’t delete) the old ISO-TP library and CanController code behind a build flag

- Update the web flasher tool to set CANopen node-id instead of raw CAN-id

- Write an EDS file for the UC2 device profile; distribute it alongside firmware releases

- Update Python tooling (UC2Client) to optionally speak CANopen via python-canopen on the Pi side

# 7. Files Changed / Not Changed

| File / Directory | Change | Details |
| --- | --- | --- |
| main/src/can/ | REPLACE | New CANopenModule.h/.cpp replaces CanController + ISO-TP code |
| lib/ESP-CAN-ISO-TP-TWAI/ | DEPRECATE | Keep behind build flag for legacy boards |
| lib/CANopenNode/ (new) | ADD | Git submodule to CANopenNode + ESP32 port |
| main/src/can/OD.h, OD.c (new) | ADD | Generated Object Dictionary for UC2 profile |
| main/ModuleController.cpp | MINOR | Add CANopenModule to the module map (same pattern as existing modules) |
| platformio.ini | MINOR | New build environments; add CANopenNode lib_deps; add -DCAN_CONTROLLER_CANOPEN flag |
| PinConfig.h | NONE | CAN_RX/CAN_TX already defined — no change needed |
| All other modules | NONE | Motor, LED, Laser modules untouched — they still respond to act()/get() |
| Serial/WiFi handlers | NONE | JSON API over serial/WiFi continues to work as before |

# 8. Raspberry Pi Side: CANopen Master Options

On the Pi, you have two options for the CANopen master that communicates via socketCAN through the CAN HAT:

## 8.1 Option A: python-canopen (Recommended for UC2)

The python-canopen library is a mature CANopen master implementation for Python. It speaks natively to your existing Python toolchain (UC2Client, ImSwitch). The Pi loads the EDS file, discovers nodes via NMT, reads/writes OD entries via SDO, and maps PDOs for real-time motor position feedback. Firmware updates become a Python script that reads a .bin file and performs SDO block transfer to the target node.

## 8.2 Option B: CANopenLinux (For Advanced Use)

If you later need a native C CANopen master on the Pi (e.g., for a standalone embedded deployment without Python), the CANopenLinux project provides a full NMT master + SDO client over socketCAN. This is the same CANopenNode stack running on the Pi side.

# 9. Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Flash size increase | CANopenNode adds ~30–50 KB. ESP32-S3 with 8 MB flash has plenty of headroom. | Disable unused CO modules (CO_config.h) — e.g., skip SRDO, GFC if not needed. |
| RAM overhead | OD + CO objects need ~5–10 KB RAM depending on OD size. | Keep OD minimal. Use DOMAIN type for large transfers (streamed, not buffered). |
| Arduino + ESP-IDF conflicts | CANopenNode_ESP32 is pure ESP-IDF. May conflict with Arduino TWAI wrappers. | Use the dual-framework approach you already use. CANopenNode accesses TWAI via ESP-IDF directly; Arduino layer untouched. |
| Existing deployments | Devices in the field use old ISO-TP protocol. | Build flag (#ifdef) keeps old path compilable. Phase migration per device. |
| Learning curve | CANopen has many concepts (NMT, SDO, PDO, OD, EDS). | Start with Phase 1 PoC. The skpang trainer + CANopenDemo tutorial cover the basics. |

# 10. Summary: What Changes, What Doesn’t

Changes: The CAN transport layer swaps from custom ISO-TP framing to CANopen (NMT + SDO + PDO). A new CANopenModule class bridges OD entries to your Module interface. The Raspberry Pi gains standard CANopen tooling (python-canopen, candump, EDS files). Firmware updates over CAN become a standard SDO block transfer.

Does NOT change: ModuleController, all hardware modules (Motor, LED, Laser, etc.), the JSON REST API, Serial/WiFi communication, PinConfig, the web flasher, the “same firmware for all satellites” principle, or any of the Python tooling that talks over serial. The integration is additive and guarded by build flags.

This is a targeted, incremental integration that gives you an industry-standard CAN protocol stack with firmware update capability, while preserving everything that already works.